#Pins:

#Raspberry Pi Pin NFC RFID Module

#Pin 6 = GND

#Pin 2= VCC

#Pin 3 =SDA

#Pin 5 =SCL

#Enter the following Commands:

#sudo raspi-config (enable i2c)

#[interface -> enable i2c->finish)

#pip3 install adafruit-circuitpython-pn532 --break-system-packages

#Enter the following Python Programs:in thorny

#Program 1: to verify the card number

import board

import busio

from adafruit_pn532.i2c import PN532_I2C

i2c = busio.I2C(board.SCL, board.SDA)

pn532= PN532_I2C(i2c)

ic, ver, rev, support = pn532.firmware_version print(f"successs! Found PN532 with Fireware: {ver}.{rev}")

print("Place your blue fob or white card on the red board...")

while True:

uid= pn532.read_passive_target(timeout=0.5)

if uid is not None:

print(f"Found Tag! ID is : {[hex(i) for i in uid]}")

#Program 2: Write the data

import board

import busio

from adafruit_pn532.i2c import PN532_I2C

# Setup I2C

i2c = busio.I2C(board.SCL, board.SDA)

pn532 = PN532_I2C(i2c)

print("--- NFC Writer ---")

text_to_write = input("Enter a small message to save to the card: ")

# Data must be exactly 16 bytes for a single block

# We pad the text with spaces if it's too short

data = text_to_write.ljust(16).encode()

print("Now, place your white card on the reader...")

while True:

uid = pn532.read_passive_target(timeout=0.5)

if uid is not None:

try:

# We use block 4 (Sector 1) to avoid messing with system blocks # This requires a 'default' key for Mifare cards

key = b'\xFF\xFF\xFF\xFF\xFF\xFF'

if pn532.mifare_classic_authenticate_block(uid, 4, 0x60, key):

pn532.mifare_classic_write_block(4, data)

print(f"Success! '{text_to_write}' written to card.")

break

else:

print("Authentication failed!")

except Exception as e:

print(f"Error: {e}")

break

#Program 3: Read the data

import board

import busio

from adafruit_pn532.i2c import PN532_I2C

i2c = busio.I2C(board.SCL, board.SDA)

pn532 = PN532_I2C(i2c)

print("Waiting for card to read stored data...")

while True:

uid = pn532.read_passive_target(timeout=0.5)

if uid is not None: key = b'\xFF\xFF\xFF\xFF\xFF\xFF'

if pn532.mifare_classic_authenticate_block(uid, 4, 0x60, key):

data = pn532.mifare_classic_read_block(4)

if data is not None:

print(f"Stored Data: {data.decode().strip()}")

break

#Q2. Write a program to display unique ID for the input data using RFID module.

#Command:

#Command 1: sudo raspi-config (enable i2c)

#Command 2: sudo reboot

#Command 3: pip3 install adafruit-circuitpython-pn532 --break-system-packages

#Command 3: sudo apt install -y libnfc-bin libnfc-dev libusb-dev libpcsclite-dev i2c-tools

#Command 4: sudo nano /etc/nfc/libnfc.conf

#device.name = "PN532 over I2C"

#device.connstring = "pn532_i2c:/dev/i2c-1"

#Save the file.

#Command 5:

#i2cdetect –y 1 (optional put sudo)

#Command 6:

#nfc-list

#Enter the following Python program in thonny.

import subprocess

import time

NAME = "Upasana" # Put your name here

last_uid = None

try:

while True:

output = subprocess.getoutput("nfc-list")

if "UID" in output:

for line in output.splitlines():

if "UID" in line:

uid = line.split(":")[1].strip().replace(" ", "")

if uid != last_uid:

print(f"{NAME}: {uid}")

last_uid = uid

break time.sleep(1)

except KeyboardInterrupt:

print("\nStopped")